.cHARDTBLS.DOC

As used by ADC, the 11 byte TurboDOS hard disk Disk Specification Table is

incorporated into a 16 byte per drive block called HDTBLx which is located in

high memory on the master processor.  The block is constituted as shown below:

Byte      Symbol		 Description

0		BS	block size (3=1K, 4=2K,	   , 7=16K)

1-2		TB	total number of blocks on disk

3		DB	number of directory blocks

4		SS	sector size (0=128, 1=256,...., 7=16K)

5-6		SC	number of sectors per track (cylinder)

7-8		TD	number of tracks on the disk

9-10	BT	number of reserved (boot) tracks

11		SR	step rate (see HDC-2001 manual)

12		HO	head offset (for drive partioning)

13		PD	physical drive (HDC port #)

14-15	ST	sectors per track (per surface)

EXAMPLES:

#�� � �� �� �� �à�� �Ġ�� �Ԡ��*�� �� �� �Ԡ��

MAX85-1	    85 00 08 08 02 10 00 00 04 00 00 80 00 00 10 00

MAX85-1A    84 00 10 20 02 10 00 00 04 00 00 80 01 00 10 00

MAX85-3	    85 00 18 18 02 30 00 00 04 00 00 80 02 00 10 00

MAX85-2	    85 00 10 10 02 20 00 00 04 00 00 80 05 00 10 00

MAX85-1/3   84 50 05 10 02 10 00 54 01 00 00 80 07 00 10 00

MAX85-2/3   84 50 05 10 02 10 00 A9 02 55 01 80 07 00 10 00

MAX85-3/3   84 50 05 10 02 10 00 00 04 AA 02 80 07 00 10 00

SQ312	    04 38 13 20 01 40 00 67 02 00 00 C0 00 01 20 00

TM503	    85 4C 0E 10 02 60 00 32 01 01 00 00 00 01 10 00

TM603SE	    85 BC 0A 10 02 60 00 E6 00 01 00 0A 00 01 10 00

MAX85 is a single physical drive partitioned into 7 logical drives by heads for

1, 1A, 3, 2, & x/3 and by tracks amongst x/3.  The allocation block sizes as

well as directory sizes vary.

# - high bit set indicates fixed media drive

* - high bit set indicates Error Correction enabled

.e

########################